Kansas Carbonate Rock Sample Information

- Twelve rock samples were used in this study. All samples were reservoir rocks, predominantly carbonate in mineralogy, obtained from core plugs and from various locations in Kansas. Eight grainstones were from the Lansing-Kansas City Group (Upper Pennsylvanian in age). Two packstones and two mudstones were from the Arbuckle Group (Cambro-Ordovician in age). Seven of the grainstones (samples 115, 92, 81, 151, 53, 135, and 89) were from various oil and gas fields in central Kansas, and one grainstone (sample 16) was from southwest Kansas. All packstones and mudstones (samples 13-18, 14-9, 16-15, and 33-25) were from the Wellington field in south Kansas. Samples were chosen over a range of porosities and permeabilities obtained from routine core analysis by commercial laboratories. This information is summarized in Table 1. We cut the core plugs into thin sticks (roughly 1.5 mm – 3.5 mm thick) to prepare for micro-CT scanning. Samples cut into thin sticks (1.5 – 3 mm thick). Micro-CT scanning was performed at the KU IBER with the Xradia MicroXCT-400; 4x and 10x lens magnifications used; Acquired 1000 image “slices” for all samples, roughly 1000x1000 pixels each.

All of the samples were performed with pore network modeling

| Sample | Sample - Rename | General Location in Kansas | Depth (ft) | Group | Period | Lithology | Helium Porosity (%) | Permeability, Kair (mD) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 115 | G1 | Central | 2956 | Lansing-Kansas City | Upper Pennsylvanian | Oomoldic Grainstone | 32.6 | 16.1 |
| 92 | G2 | Central | 3507 | Lansing-Kansas City | Upper Pennsylvanian | Oomoldic Grainstone | 32.2 | 11.9 |
| 16 | G3 | Southwest | 4600 | Lansing-Kansas City | Upper Pennsylvanian | Oomoldic Grainstone | 26.4 | 1.1 |
| 81 | G4 | Central | 3057 | Lansing-Kansas City | Upper Pennsylvanian | Oomoldic Grainstone | 25.6 | 50.4 |
| 151 | G5 | Central | 3099 | Lansing-Kansas City | Upper Pennsylvanian | Oomoldic Grainstone | 20.5 | 3.5 |
| 53 | G6 | Central | 3196 | Lansing-Kansas City | Upper Pennsylvanian | Oomoldic Grainstone | 14.0 | 19.3 |
| 135 | G7 | Central | 3132 | Lansing-Kansas City | Upper Pennsylvanian | Oomoldic Grainstone | 8.3 | 0.03 |
| 89 | G8 | Central | 3011 | Lansing-Kansas City | Upper Pennsylvanian | Oomoldic Grainstone | 5.7 | 0.01 |
| 13-18 | P1 | South | 4203 | Arbuckle | Cambro-Ordovician | Packstone | 7.7 | 27.9 |
| 14-9 | P2 | South | 4253 | Arbuckle | Cambro-Ordovician | Packstone | 2.8 | 198.9 |
| 16-15 | M1 | South | 4309 | Arbuckle | Cambro-Ordovician | Mudstone | 3.2 | 0.07 |
| 33-25 | M2 | South | 5074 | Arbuckle | Cambro-Ordovician | Mudstone | 11.8 | 5.8 |

Our study utilized X-ray micro-computed tomography (micro-CT) scanning technology to obtain three-dimensional digital rock volumes. The Xradia MicroXCT-400 microscope at the University of Kansas Bioengineering Research Center (BERC) uses phase contrast micro-CT. X-rays travel through the sample and refract according to Snell’s law. The resulting image intensity, or brightness, is high in areas where x-rays are converging, and vice-versa for low intensity, or darker, areas. The detector measures the phase contrast. In effect, brighter regions indicate harder rock parts, and darker regions indicate softer rock parts and pores. This brightness contrast is the basis for identifying solid versus porous regions during the segmentation process. In our study, we used microscope lenses of two different optical magnifications, 10X and 4X, to explore different resolutions over the samples. Eight samples were scanned with the 4X lens, and four were scanned with the 10X lens. There is a trade-off between the magnification level and the effective volume that can be scanned; higher resolution scans cover a smaller 3D volume. On average, we achieved resolutions of 1.81 micrometers per pixel with the 10X lens and 4.30 micrometers per pixel with the 4X lens.

| Sample | Magnification Lens | Pixel Length | Voxel Volume |  |
| --- | --- | --- | --- | --- |
| 115 | 4X | 4.30 |  |  |
| 92 | 4X | 4.30 |  |  |
| 16 | 10X | 1.92 |  |  |
| 81 | 4X | 4.30 |  |  |
| 151 | 4X | 4.30 |  |  |
| 53 | 4X | 4.30 |  |  |
| 135 | 4X | 4.30 |  |  |
| 89 | 10X | 1.95 |  |  |
| 13-18 | 10X | 1.79 |  |  |
| 14-9 | 4X | 4.30 |  |  |
| 16-15 | 10X | 1.62 |  |  |
| 33-25 | 4X | 4.30 |  |  |

Colgin MS Thesis documented the choice of simulation parameters.

Refer to “JColgin_Thesis_Defense.pptx”

For Fan Zhang’s SIP simulation work on carbonate rocks involving three polarization mechanisms (mainly done in 2017 and 2018), please refer to “Zhang_SEG_2018_oral_final”, his simulation fits the experimental data quite well, especially for LKC-53, LKC-59, and LKC-92, see below